ERW modeling questions:

- What is the source of data on the basalt feedstock locations? Any alternative datasets? What other assumptions should we consider?

- What fixed and variable components of transportation costs?

- Can we explicitly say what assumed other costs are (Grinding + feedstock + spreading +?)? maybe a table

- What is the MRV assumption, explicitly?

- What is the definition of “durable” CDR here?

- On slide 8: Durable CDR (Mt) is the Millions of metric tons of carbon dioxide removed? % treated is the percentage of what exactly?

- On slide 9: what is the reference for this assumption: durable CDR  =  max( gross CDR  −  F · S ,  0 )? Pls explain “F = 0.88 tCO₂ per tCaCO₂-eq    ”

- Slide 10: Describe the Ecocrop model briefly. What crops are we evaluating acidity responses to? Are we using pH or exchangeable acidity or something else?

- Slide 10: Explain buffered shift: “pH change is damped by soil buffering (β = kβ·ECEC); reacted alkalinity A = gross CDR / F.” Explain sensitivity: “Sensitivity A low–high band over (kβ, realized-alkalinity) is carried — the uniform-50 case is where it bites hardest.”

- What is the source of our assumptions on kinetic saturation? What reference?

- Slide 12: explain “Equilibrium - Steady-state durable removal — the headline regime.” this sounds like AI-speak

# Prep for hard questions - ERW ex ante model

Grounded in the committed code (erw/erw-*.R, paper/analysis/) and the manuscript. Each item gives the short answer, the specifics, and - where it matters - the honest weak spot and how I'd respond. Numbers are the equilibrium, targeted, net-export headline unless noted.

## A. Data and cost assumptions

### 1. Source of basalt feedstock locations; alternatives; other assumptions

Source. Feedstock source geology comes from the GLiM - Global Lithological Map (Hartmann & Moosdorf 2012; PANGAEA doi:10.1594/PANGAEA.788537). We rasterize three mafic classes as basalt-equivalent feedstock: vb basic volcanic (basalt s.s.), py pyroclastics (basaltic tuff), and pb basic plutonic (gabbro). Ultramafic classes are deliberately excluded for cropland use because of Ni/Cr leaching risk. erw-basalt-access.R / erw-build-basalt-mask.R.

These GLiM polygons are the origin points; delivered cost is then a cost-distance from every cropland pixel to the nearest source (see Q2).

Alternatives we could/should offer.

- National / regional geological surveys at finer resolution than GLiM (GLiM is coarse and lumps lithologies); would sharpen source locations, especially in the East African Rift.

- Active-quarry point inventories (the code already supports an optional quarry-point mask). Real ERW would source from operating quarries with crushing capacity, not from every basalt outcrop - this is arguably the more economically honest source set.

- Mine-tailings / by-product silicate streams (steel slag, mine waste) as alternative feedstocks - different chemistry, different geography.

Other assumptions worth stating up front. A single "representative basalt" chemistry (CaO 10%, MgO 7%, plus K/Na/P; Ni 100 ppm, Cr 200 ppm) is applied everywhere - we do not vary feedstock chemistry by source. Grain size is fixed at 50 µm. Both are decision levers we hold constant in the headline and could sweep.

Weak spot / response. "GLiM outcrops ≠ deliverable supply." True - we map geological availability, not quarry capacity or permitting. The transport layer partly compensates (remote sources are expensive), but a production version should intersect with an active-quarry inventory. Frame the current map as an upper bound on feedstock access.

### 2. Fixed vs variable components of transportation cost

Transport is modeled as purely variable (distance/time-proportional):

transport_cost ($/t) = travel_time(min) × $0.04/min/t

- $0.04/min/t = a 30-tonne truck at ~$80/hr operating cost (USD_PER_MIN_PER_T).

- travel_time = accumulated cost-distance from the nearest basalt source over the Malaria Atlas Project friction surface (Weiss et al. 2020, nominal 2019), which folds in roads, terrain, rivers, urban slow-downs. terra::costDist.

- Haul distance for the LCA is travel_time × 0.5 km/min (≈30 km/h effective). Pixels beyond a 1,000 km cap are treated as unreachable.

The genuinely fixed (per-tonne, location-independent) costs live elsewhere in the stack: quarry-gate feedstock ($10/t), grinding (set by grain size and local power price), and spreading ($8/t). See the Q3 table.

Weak spot / response. There is no separate fixed loading/handling charge, no backhaul/return-trip accounting, no rail option, and no trucking economies of scale - it's a linear per-tonne-per-minute haulage cost. That likely overstates marginal cost for long, high-volume, road- or rail-served hauls and understates the fixed cost of short hauls. Honest framing: a transparent first-order haulage proxy; the sensitivity analysis should sweep $/min/t because delivered cost is the dominant swing variable.

### 3. Explicit "other costs" (grinding + feedstock + spreading + ?) - table

Delivered basalt cost is decomposed per tonne (erw-7, erw-3, erw-basalt-access):

| Component | Value (reference) | Type | Basis / source |
| --- | --- | --- | --- |
| Ex-quarry feedstock (quarry gate) | $10 / t | fixed per t | basalt-fines aggregate by-product price (QUARRY_GATE_USD_T) |
| Grinding to 50 µm | ≈$1.94 / t at $0.10/kWh; per-pixel = 19.4 kWh/t × local electricity price | fixed per t (varies by country power price) | Strefler et al. 2018 grinding-energy curve; 0.07 GJ/t = 19.4 kWh/t at 50 µm |
| Transport (haulage) | travel_time × $0.04/min/t (spatial) | variable (distance) | 30-t truck @ ~$80/hr over MAP friction surface (Q2) |
| Spreading (on-farm application) | $8 / t | fixed per t | SPREADING_USD_T |
| = Delivered cost / t basalt | quarry + grind + transport + spread |  | consumed by erw-7 as basalt_cost_per_t |
| MRV (separate) | $20 / tCO₂ | per tCO₂, netted from carbon price | see Q4 |

Two things are handled as CO₂, not dollars, and are subtracted from gross removal (lifecycle emissions, kg CO₂/t basalt): grinding (kWh/t × grid carbon intensity, country raster; fallback 0.60 kgCO₂/kWh), transport (km × 0.12 kgCO₂/t-km, diesel truck), and spreading (0.5 kgCO₂/t).

A cost table like this belongs in the paper (or SI) - it's the clearest way to preempt Q2-Q4 and it's currently only in prose/comments.

Weak spot. Grinding-plant capex, quarry royalties, storage, and financing are not itemized - they are implicitly inside the $10 quarry gate and the energy-based grinding proxy. The $10/t is optimistic (assumes fines are a cheap by-product); worth a sensitivity row.

### 4. The MRV assumption, explicitly

MRV = $20 per tCO₂, subtracted directly from the carbon price so revenue = net_CDR × (carbon_price − MRV) = net_CDR × ($150 − $20). MRV_COST_USD_T_CO2 in erw-7.

The stack it represents: field sampling, lab analysis, verification, and registry fees. Code comment records the reasoning: $30/tCO₂ is the first-of-kind pilot benchmark (Levy et al. 2024); $20 is the at-scale aggregator/pooling target (Isometric/Puro-type protocols, 2027+). We use the optimistic-but-defensible $20 in the headline.

Weak spot / response. Real early-market ERW MRV (dense soil/water sampling, uncertain reactive-transport models) can run far above $20 - some estimates put first-mover MRV at $50-100+/tCO₂. Since MRV enters one-for-one with price, it's a first-order lever. Response: hold it explicit, sweep $20/$30/$50, and note the headline uses the mature-market value.

## B. Carbon accounting (slides 8-9)

### 5. Definition of "durable" CDR

Two deductions, applied in sequence, define what we credit:

- Gross CDR - CO₂ consumed by basalt weathering, scaled by a climate (temperature/precipitation) factor and a small pedogenic-carbonate loss in arid zones (erw-9).

- Net-export CDR = max(gross − F·S, 0) - subtract the alkalinity consumed neutralizing soil acidity, which re-releases its CO₂ (Q7). This is the "durable" quantity: alkalinity that leaves the field as bicarbonate (HCO₃⁻) with drainage and is stored in rivers/ocean for >10,000 years (Renforth 2019; Kanzaki et al. 2023). It is durable in the geochemical-permanence sense.

- Net (creditable) CDR = net-export − lifecycle supply-chain emissions (grinding/transport/spreading CO₂). Revenue is on this figure.

So "durable" = removal that survives both the acidity re-release and the supply-chain emissions - carbon that is actually gone for millennia.

Weak spot. ">10,000 years" is the ocean-alkalinity permanence argument; it assumes the bicarbonate reaches and stays in drainage/ocean. Riverine/estuarine CO₂ evasion (some HCO₃⁻ degasses en route) is not modeled and would trim durability further.

### 6. Slide 8 - units of "Durable CDR (Mt)" and "% treated"

- Durable CDR (Mt) = millions of tonnes of CO₂ durably removed per year (net-export basis, equilibrium regime), summed over the area that is public-viable at that carbon price. Metric tonnes CO₂, not tonnes C.

- % treated = share of the 17.6 Mha of "treated cropland" - the acid-eligible cropland that receives the targeted (lime-equivalent) dose (defined in Q12). It is not all cropland. The manuscript also reports the all-cropland share (of 146 Mha, ~one-eighth of the treated share) for reference. So "$150 → 9.4% treated" means 1.65 Mha of the 17.6 Mha treated base, ≈1.1% of all SSA cropland.

### 7. Slide 9 - reference for durable = max(gross − F·S, 0), and F = 0.88

The equation is mass balance, not a fitted parameter. It's our own derivation (documented in paper/netexport-cdr-memo.md), resting on the standard geochemistry that neutralizing acidity re-releases CO₂ exactly as agricultural lime does (Hartmann et al. 2013; the lime analogy is textbook). The acidity sink S is the soil's lime requirement in t CaCO₃/ha, computed by erw-3 via the LiTAS/Kamprath method (Aramburu-Merlos et al. 2023).

F = 0.88 tCO₂ per t CaCO₃-equivalent falls straight out of the feedstock's own constants:

F = CDR_eff / effective_NV

= 87.6 kg CO2 per t basalt  ÷  99.6 kg CaCO3-eq per t basalt

= 0.88 tCO2 / tCaCO3-eq

Plainly: one tonne of our basalt removes 87.6 kg CO₂ and supplies 99.6 kg of acid-neutralizing capacity. So each unit of neutralizing capacity "carries" 0.88 units of CO₂ - and when that capacity is spent fighting soil acidity instead of exporting, that 0.88 is re-released. S is per-regime: standing exchangeable acidity (caco3_kamprath) for year-1/NPV, annual maintenance acidity (caco3_merlos_maintenance) for equilibrium.

Important reproducibility note (be ready for this). erw-9 writes both gross (cdr_yield_*.tif) and net-export (cdr_yield_*_netexport.tif) rasters. The headline net-export envelopes were produced by the analysis engine that applies max(gross − F·S,0) directly; erw-7 as currently committed reads the gross rasters. Before submission we should repoint erw-7/erw-11 at the _netexport layers so the R pipeline reproduces the headline end-to-end. If asked "does the committed pipeline produce the net-export numbers in one run?" - the honest answer is "the deduction is implemented in erw-9 and the analysis engine; wiring erw-7 to consume it is a cleanup item on the reproducibility checklist."

Weak spot we already flag (memo §6). This is a first-order sequential bound - it assumes the acidity sink is fully paid down before any alkalinity exports. Reality is simultaneous: some carbon exports even on acid soils, and re-acidification keeps regenerating the sink. Truth lies between "gross" (old) and "net" (this). We present both as bounds and lead the carbon case on equilibrium, where they converge.

## C. Agronomic response (slide 10) and kinetics (slides 10-11)

### 8. The EcoCrop model; which crops; pH vs exchangeable acidity

EcoCrop is FAO's crop environmental-requirements database; we run it through the R package Recocrop. It maps an environmental variable to a relative yield (0-1) using a piecewise-linear/trapezoidal response defined by breakpoints (absolute min, optimum-lower, optimum-upper, absolute max). erw-4, erw-5.

Crops (23, all SPAM): maize, sorghum, common bean, chickpea, lentil, wheat, barley, arabica & robusta coffee, pearl & finger millet, potato, sweet potato, cassava, cowpea, pigeon pea, soybean, groundnut, sugarcane, cotton, cocoa, tea, tobacco.

Which variable - the key point: the headline model runs on acidity saturation (exchangeable acidity as % of ECEC - effectively aluminum saturation), not pH. This is the mechanistically correct driver of acid-soil yield loss (Al³⁺ toxicity), and it's what the targeting and yield-loss layers use (resp_hp, soil$hp_sat vs per-crop ac_sat tolerance). We also compute a pH-based response as an alternative/robustness layer. Yield loss is clamped to [0.2, 1] (we never let ERW "create" more than a 5× yield or a full crop failure).

Weak spot / response (a sharp coauthor may catch this). The over-liming penalty prototype (Refinement 1) was built on the EcoCrop pH ceiling (post-application pH pushed past the crop optimum), whereas the core envelope model runs on acidity saturation. That's a mild internal inconsistency of variable. Defensible response: over-liming (raising pH past optimum, inducing micronutrient lock-up) is genuinely a pH phenomenon, so using the pH ceiling for the penalty is appropriate even though the baseline yield gain is driven by relieving Al saturation - but we should state this explicitly and, ideally, express both on a common axis.

### 9. Slide 10 - buffered shift and the sensitivity band (Refinement 1)

Buffered shift. How far pH moves for a given dose of reacted alkalinity A:

final_pH ≈ pH0 + A / β, where β = kβ · ECEC and A = gross_CDR / F.

- A = reacted alkalinity (t CaCO₃-eq/ha), backed out of gross CDR by the same factor F.

- β = the soil's buffering capacity - how much alkalinity it takes to move pH one unit. Higher ECEC (cation exchange capacity) → more buffered → smaller pH move per tonne. kβ is the (uncertain) proportionality between ECEC and buffering.

- Once the acidity demand is met (A > S), extra alkalinity pushes pH above neutral toward the crop's ceiling; the EcoCrop pH response then applies a yield haircut past the optimum. That's the over-liming penalty.

Sensitivity band. Because both kβ (buffering) and the realized-alkalinity multiplier on A are uncertain, we carry a low-high band over them: low penalty = (mA=0.5, kβ=2.0), central = (1.0, 1.0), high = (1.5, 0.5). "Uniform-50 is where it bites hardest" because a blanket 50 t/ha dose massively over-applies relative to the lime requirement on most pixels, so pH is driven far past the optimum: the private return there falls from $997M (no penalty) to $814M central and ~$8M in the high scenario - i.e., aggressive uniform over-liming can erase the private return, while the targeted dose is barely touched (4.31 → 4.02 Mha, −7%). This is the intended message: moderate, targeted doses are robust; blanket-high doses are fragile.

Weak spot. kβ is not calibrated to SSA soils - it's a plausible range, not a measured relationship. Say so.

### 10. Source of the kinetic-saturation assumption (Refinement 2)

Honest answer: this is a functional-form stress test, not a calibrated relationship. The multiplier sat(D) = (Kd + 50)/(Kd + D) encodes the well-documented qualitative finding that per-tonne weathering efficiency declines as application rate rises (a given soil column, moisture, and CO₂ supply can only weather so much rock per year). It is anchored to Lewis et al. (2021), whose ~50 t/ha experiment we use as the reference dose where sat = 1, and to the general saturation behavior seen in mesocosm/column studies (e.g., Strefler et al. 2018 on grain-size/rate trade-offs). The half-saturation constant Kd itself is uncalibrated, which is exactly why we report a band (Kd = ∞/60/30/15) rather than a point estimate.

Why this matters for the story (and the likely follow-up). We show that saturation makes CDR concave in dose (creating an interior optimum) but does not drive the public envelope to zero - below the 50 t/ha reference, normalized per-tonne reactivity is actually higher, so at targeted doses durable CDR rises. The binding constraint on the carbon case is MAC > price (median MAC $446/tCO₂ vs $150 price), not kinetics. If pushed - "so you can't cite a number for Kd?" - concede it's uncalibrated and frame it as sensitivity, then pivot to the MAC point, which does not depend on it.

### 11. Slide 12 - "Equilibrium: steady-state durable removal" in plain language

Yes, that bullet reads like jargon. Here's the plain version to use:

The model runs three timeframes. Year-1 is the first application on soil that still carries its full historical acidity load. Equilibrium is the long-run maintenance phase: picture a field whose acidity has already been neutralized (by past liming or earlier ERW), so each year you only add enough basalt to offset that year's fresh acidification - from fertilizer, crop removal, and rainfall. In that maintenance state, very little of the weathering is "spent" fighting a backlog of acidity, so most of the alkalinity exports as bicarbonate and counts as durable carbon removal.

We headline equilibrium because year-1 CDR is dominated by paying down a one-time acidity debt (mostly re-released, per Q7), whereas the maintenance/steady state is what an ongoing ERW program actually looks like. Retention illustrates it: year-1/NPV retains only ~19-23% of gross CDR as durable; equilibrium retains ~73-87% (net-export memo §4). Swap the slide wording to: "the ongoing maintenance phase, once soils are already at target - when most weathering goes to carbon rather than to clearing an acidity backlog."

## D. The typology map (slide 14)

### 12. The universe of candidate (non-white) pixels

Non-white pixels = the ~17.6 Mha of "treated cropland": acid-eligible cropland that receives the targeted dose. Precisely, a pixel is in the universe if, for at least one of the 23 SPAM crops grown there, both:

- SPAM harvested area > 0 (the crop is actually grown on that pixel), and

- soil acidity saturation exceeds that crop's tolerance (soil$hp_sat > crop ac_sat) - i.e., the soil is acid enough that liming/ERW is agronomically indicated, so a LiTAS lime-equivalent dose is prescribed.

erw-7 builds the per-crop mask (area_mask × acid_mask); erw-11 then keeps every pixel where the area-weighted combined margin is defined (wsum > 0) and classifies it into private / public / intersection / combined-only / neither. Values are area-weighted across the crops sharing the pixel.

White pixels are therefore: non-cropland; cropland that is not acid enough to trigger the targeted dose for any crop it grows; or pixels with no defined economics (missing price/soil/CDR). White ≠ "ERW fails" - most white is simply "not acid-eligible" or "not cropland."

Contrast with the uniform-rate maps: under uniform 10/20/50 the universe is all cropland regardless of acidity (matching the Beerling/Baek convention), so those maps have a larger non-white footprint. Slide 14 is the targeted, equilibrium map, so its universe is the acid-eligible 17.6 Mha.

Weak spot / response. The acid-eligibility threshold is crop-specific and depends on the SoilGrids acidity-saturation layer and the EcoCrop ac_sat tolerances - both uncertain. A reviewer could argue the 17.6 Mha denominator is itself a modeling choice. Fair; it's why we always report shares of both treated (17.6 Mha) and all cropland (146 Mha).

## Two cross-cutting points to have ready

- Everything is per-pixel and area-weighted across co-located crops - there is no single "representative farm"; farm counts are a coarse post-hoc conversion using national average holding sizes (order-of-magnitude only).

- The three levers that actually move the public envelope are carbon price, delivered cost, and durable-CDR-per-tonne (MAC = cost ÷ durable CDR). Net-export accounting and MAC > price - not kinetic saturation - are what keep the carbon-only case small at today's $150. This is the line to return to whenever a carbon question gets into the weeds.